BMWG Working Group             				 F. Calabria

Internet-Draft                                           Cisco

Intended status: Informational                    C. Pignataro

Expires: August 24, 2026         		  Blue Fern Consulting

February 2026

Benchmarking Methodology for AI Inference Serving Network Fabrics
draft-calabria-bmwg-ai-fabric-inference-bench-00

Abstract

This document defines benchmarking terminology, methodologies, and Key Performance Indicators (KPIs) for evaluating Ethernet-based AI inference serving network fabrics. As Large Language Model (LLM) inference deployments scale to disaggregated prefill/decode architectures spanning hundreds or thousands of accelerators (GPUs/XPUs), the interconnect fabric becomes the critical bottleneck determining Time to First Token (TTFT), Inter-Token Latency (ITL), and aggregate throughput in tokens per second (TPS). This document establishes vendor-independent, reproducible test procedures for benchmarking fabric-level performance under realistic AI inference workloads.

Coverage includes RDMA-based KV cache transfer between disaggregated prefill and decode workers, Mixture-of-Experts (MoE) expert parallelism AllToAll communication, request routing and load balancing for inference serving, congestion management under bursty inference traffic patterns, and scale/soak testing. The methodology enables apples-to-apples comparison across implementations, NIC transport stacks (RoCEv2, UET), and fabric architectures.

This document is a companion to draft-bmwg-ai-fabric-training-bench-00, which addresses training workloads, and the associated terminology document that SHOULD be consulted before applying these methodologies.

Status of This Memo

This Internet-Draft is submitted in full conformance with the provisions of BCP 78 and BCP 79. Internet-Drafts are working documents of the Internet Engineering Task Force (IETF). Note that other groups may also distribute working documents as Internet-Drafts. The list of current Internet-Drafts is at https://datatracker.ietf.org/drafts/current/.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time. It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress."

Copyright Notice

Copyright (c) 2026 IETF Trust and the persons identified as the document authors. All rights reserved. This document is subject to BCP 78 and the IETF Trust's Legal Provisions Relating to IETF Document (https://trustee.ietf.org/license-info) in effect on the date of publication of this document.

Table of Contents

1. Introduction

1.1. Requirements Language
1.2. Scope and Applicability
1.3. Relationship to Existing BMWG Work
1.4. Relationship to Companion Training Document

2. Terminology and Definitions

3. Test Topology and Architecture

3.1. Reference Fabric Topologies
3.2. Disaggregated Prefill/Decode Topology
3.3. Device Under Test (DUT) Identification
3.4. Traffic Generator and Workload Emulator Requirements

4. KPI Framework and Metrics Taxonomy
4.1. Primary Latency KPIs
4.2. Primary Throughput KPIs

4.3. Fabric-Level KPIs
4.4. Fabric Health Indicators

5. Test Category 1: RDMA KV Cache Transfer Benchmarks

5.1. Point-to-Point KV Cache Transfer Throughput
5.2. KV Cache Transfer Latency
5.3. Concurrent KV Cache Transfer Scaling
5.4. Multi-Tier Storage Transfer Characterization

6. Test Category 2: Prefill/Decode Disaggregation Benchmarks
6.1. End-to-End Disaggregated TTFT

6.2. xPyD Ratio Optimization
6.3. Heterogeneous Parallelism Configuration
6.4. Prefill Queue Depth Impact on Transfer Latency

7. Test Category 3: MoE Expert Parallelism Benchmarks

7.1. AllToAll Dispatch Throughput
7.2. Normal vs. Low-Latency Dispatch Mode Comparison
7.3. Wide Expert Parallelism Scaling
7.4. Expert Parallelism and KV Cache Transfer Contention

8. Test Category 4: Congestion Management Benchmarks

8.1. ECN Marking Under Inference Incast
8.2. PFC Behavior Under Bursty KV Cache Transfers
8.3. Congestion Control Convergence for Mixed Traffic8.4. PFC Storm and Deadlock Resilience

9. Test Category 5: Request Routing and Load Balancing

9.1. KV-Aware Request Routing Efficacy
9.2. Prefix-Aware Cache Hit Rate
9.3. ECMP and Dynamic Load Balancing Under Inference Traffic
9.4. Jain Fairness Index for Decode Worker Utilization

10. Test Category 6: Latency Benchmarks

10.1. TTFT Under Varying Prompt Lengths
10.2. ITL Characterization and Tail Latency
10.3. End-to-End Latency Under Multi-Tenant Load
10.4. Latency Sensitivity to Fabric Congestion

11. Test Category 7: Throughput Benchmarks

11.1. Aggregate Tokens Per Second
11.2. Batch Size Scaling and Continuous Batching Impact
11.3. Goodput Under Preemption and Eviction

12. Test Category 8: Scale and Autoscaling

12.1. Fabric Scale Limits for Inference Clusters
12.2. Dynamic Autoscaling Response Time
12.3. Link Failure Convergence Impact on Serving

13. Test Category 9: Soak and Stability

13.1. 24-Hour Sustained Inference Load
13.2. KV Cache Memory Leak Detection
13.3. Long-Running Serving Stability

14. Reporting Format

15. Security Considerations

16. IANA Considerations

17. References

17.1. Normative References
17.2. Informative References

Appendix A: KPI-to-Test Mapping Summary

Appendix B: Inference Framework Comparison

Appendix C: KV Cache Transfer Frame Format

Appendix D: MoE AllToAll Communication Pattern

Authors' Addresses

1. Introduction

Large Language Model (LLM) inference serving has emerged as a dominant consumer of datacenter network capacity, with fundamentally different fabric requirements compared to training workloads. While training workloads are characterized by bulk synchronous collective operations (AllReduce, AllGather) with predictable periodicity, inference workloads exhibit bursty, latency-sensitive request/response patterns with strict Service Level Objectives (SLOs) on per-token latency and time-to-first-token.

The advent of disaggregated serving architectures, where the computationally intensive prefill phase (prompt processing) is physically separated from the memory-bound decode phase (token generation), introduces a new class of fabric-critical data movement: KV cache transfer. A single large prompt processed by a typical large-scale model generates multiple gigabytes of KV cache state that must be transferred from prefill workers to decode workers within a fraction of the target TTFT SLO.

At cluster scale with thousands of concurrent requests, this creates sustained multi-terabyte-per-second aggregate transfer demands on the fabric. Simultaneously, Mixture-of-Experts (MoE) architectures introduce expert parallelism (EP), which distributes expert sub-networks across GPUs and requires AllToAll communication for token-to-expert routing. Wide EP configurations (e.g., 96-way EP across 12 nodes of 8 GPUs each) generate fine-grained, latency-sensitive inter-node traffic that contends with KV cache transfers on shared fabric links.

This document defines vendor-independent benchmarking methodologies for evaluating how well a network fabric supports these inference-specific traffic patterns. All tests are designed for controlled laboratory environments using either hardware traffic generators or software workload emulators capable of reproducing inference serving traffic profiles.

1.1. Requirements Language

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this document are to be interpreted as described in RFC 2119 [RFC2119] and RFC 8174 [RFC8174].

1.2. Scope and Applicability

This document addresses the benchmarking of Ethernet-based network fabrics carrying AI inference serving traffic.

The scope covers Layer 2/3 fabric performance (switch forwarding, link utilization, congestion management), RDMA transport performance (one-sided PUT/GET operations for KV cache transfer, two-sided SEND/RECV for expert parallelism dispatch), and the interaction between fabric behavior and application-level inference metrics (TTFT, ITL, TPS).

The document does NOT address benchmarking of individual accelerator (GPU/XPU) compute performance, model accuracy or quality metrics, benchmarking of the inference serving software stack in isolation from the fabric, or live production network testing. All methodologies assume controlled laboratory conditions per BMWG convention.

1.3. Relationship to Existing BMWG Work

This document builds upon the foundational BMWG benchmarking framework established by RFC 1242 (Benchmarking Terminology for Network Interconnect Devices), RFC 2544 (Benchmarking Methodology for Network Interconnect Devices), RFC 2889 (Benchmarking Methodology for LAN Switching Devices), and RFC 6349 (Framework for TCP Throughput Testing).

The test structure follows RFC 2544 conventions for trial duration (minimum 60 seconds), statistical repetition (minimum 20 trials for latency, 50 for burst), and reporting format (graphical and tabular).

The methodologies extend RFC 2544 Section 26 benchmarks (throughput, latency, frame loss rate, back-to-back frames, system recovery, reset) to inference-specific scenarios including KV cache transfer, expert parallelism dispatch, and disaggregated serving request routing.

1.4. Relationship to Companion Documents

This document is a companion to draft-bmwg-ai-fabric-training-bench-00 [TRAINING-BENCH], which defines benchmarking methodologies for AI training network fabrics. Both documents share common terminology (Section 2), test topology conventions (Section 3), and reporting formats (Section 14). Both documents SHOULD be used in conjunction with draft-bmwg-ai-fabric-terminology-00 [TERMINOLOGY], which defines the common terminology base for AI fabric benchmarking and SHOULD be consulted before applying the methodologies in either document.

Where training workloads are dominated by bulk synchronous collective communication (AllReduce, AllGather) with high bandwidth utilization and periodic synchronization barriers, inference workloads are dominated by bursty, latency-sensitive point-to-point transfers (KV cache) and fine-grained AllToAll dispatch (MoE expert parallelism) Implementers deploying converged fabrics that serve both training and inference workloads SHOULD run both test suites.

2. Terminology and Definitions

The following terminology is used throughout this document. Terms defined in the companion training document are referenced but not redefined unless the inference context introduces substantive differences.

+-------------------+--------------------------------------------------------+

| Term              | Definition                                             |

+-------------------+--------------------------------------------------------+

| TTFT              | Time to First Token. The elapsed time from receipt of  |

|                   | an inference request by the serving system to emission |

|                   | of the first output token. Includes prompt processing  |

|                   | (prefill), KV cache generation, optional KV cache      |

|                   | transfer (in disaggregated architectures), and initial |

|                   | decode step. Target: < 500 ms for interactive serving. |

+-------------------+--------------------------------------------------------+

| ITL               | Inter-Token Latency. The elapsed time between          |

|                   | successive output tokens during the autoregressive     |

|                   | decode phase. Measured at P50, P95, P99, and P99.9     |

|                   | percentiles. Target: < 50 ms P99 for interactive       |

|                   | serving.                                               |

+-------------------+--------------------------------------------------------+

| TPS               | Tokens Per Second. Aggregate throughput of the serving |

|                   | system measured as the total number of output tokens   |

|                   | generated per second across all concurrent requests.   |

|                   | Reported separately for input (prefill) TPS and output |

|                   | (decode) TPS.                                          |

+-------------------+--------------------------------------------------------+

| KV Cache          | Key-Value Cache. The intermediate attention state (key |

|                   | and value projection matrices) computed during the     |

|                   | prefill phase and reused during each decode step. Size |

|                   | scales with model dimension, number of layers, number  |

|                   | of attention heads, sequence length, and numerical     |

|                   | precision. For a 70B parameter model at FP16 with 4K   |

|                   | context: approximately 1.34 GB per request.            |

+-------------------+--------------------------------------------------------+

| Prefill Phase     | The compute-bound phase of inference in which the      |

|                   | entire input prompt is processed in parallel to        |

|                   | generate the KV cache and the first output token.      |

|                   | Characterized by high arithmetic intensity (200-400    |

|                   | ops/byte), high GPU utilization (90-95%), and large    |

|                   | activation tensors.                                    |

+-------------------+--------------------------------------------------------+

| Decode Phase      | The memory-bound phase of inference in which output    |

|                   | tokens are generated autoregressively, one token per   |

|                   | forward pass. Characterized by low arithmetic          |

|                   | intensity (60-80 ops/byte), lower GPU utilization      |

|                   | (20-40%), and memory-bandwidth-limited KV cache reads. |

+-------------------+--------------------------------------------------------+

| Disaggregated     | An inference serving architecture in which prefill and |

| Serving           | decode computations are executed on physically         |

|                   | separate groups of accelerators (workers), connected   |

|                   | by a network fabric. The KV cache generated by prefill |

|                   | workers are transferred over the fabric to decode       |

|                   | workers.                                               |

+-------------------+--------------------------------------------------------+

| xPyD Ratio        | The allocation ratio of prefill (x) to decode (y)     |

|                   | resources in a disaggregated serving cluster. For      |

|                   | example, 3P9D indicates 3 prefill nodes and 9 decode   |

|                   | nodes. The optimal ratio depends on model size, prompt |

|                   | length distribution, output length distribution, and   |

|                   | SLO targets.                                           |

+-------------------+--------------------------------------------------------+

| EP                | Expert Parallelism. A parallelism strategy for         |

|                   | Mixture-of-Experts (MoE) models in which expert        |

|                   | sub-networks are distributed across multiple GPUs.     |

|                   | Token routing to the appropriate experts requires      |

|                   | AllToAll communication.                                |

+-------------------+--------------------------------------------------------+

| Wide EP           | Expert Parallelism spanning many GPUs                  |

|                   | (e.g., 96-way EP across 12 nodes), requiring           |

|                   | inter-node AllToAll communication for every MoE layer  |

|                   | forward pass.                                          |

+-------------------+--------------------------------------------------------+

| DP Attention      | Data Parallelism applied to the attention computation, |

|                   | where the KV cache is partitioned across data-parallel |

|                   | ranks. Each rank holds 1/DP_SIZE of the KV cache, and  |

|                   | AllToAll communication is used to exchange attention   |

|                   | outputs.                                               |

+-------------------+--------------------------------------------------------+

| MoE               | Mixture of Experts. A model architecture that          |

|                   | activates only a subset of expert sub-networks for     |

|                   | each token, enabling larger model capacity with        |

|                   | sub-linear compute scaling.                            |

+-------------------+--------------------------------------------------------+

| Normal Dispatch   | A communication mode for AllToAll MoE dispatch         |

|                   | optimized for the prefill phase. Maximizes throughput  |

|                   | for long input sequences but generates dynamic         |

|                   | (symbolic) shapes incompatible with CUDA Graph.        |

+-------------------+--------------------------------------------------------+

| Low-Latency       | A communication mode for AllToAll MoE dispatch         |

| Dispatch          | optimized for the decode phase. Uses fixed input       |

|                   | shapes compatible with CUDA Graph, reducing kernel     |

|                   | launch overhead at the cost of slightly lower peak     |

|                   | throughput.                                            |

+-------------------+--------------------------------------------------------+

| RDMA              | Remote Direct Memory Access. A transport mechanism     |

|                   | enabling direct memory-to-memory data transfer between |

|                   | hosts without CPU involvement. Implementations include |

|                   | InfiniBand Verbs and RoCEv2 (RDMA over Converged       |

|                   | Ethernet v2).                                          |

+-------------------+--------------------------------------------------------+

| RoCEv2            | RDMA over Converged Ethernet version 2. An RDMA        |

|                   | transport that encapsulates InfiniBand transport over  |

|                   | UDP/IP, enabling RDMA semantics on standard Ethernet   |

|                   | fabrics.                                               |

+-------------------+--------------------------------------------------------+

| UET               | Ultra Ethernet Transport. A transport protocol defined |

|                   | by the Ultra Ethernet Consortium (UEC) Specification   |

|                   | 1.0, offering ordered/unordered reliable delivery,     |

|                   | multipath packet spraying, and integrated congestion   |

|                   | control for AI/HPC workloads.                          |

+-------------------+--------------------------------------------------------+

| KVCXL             | KV Cache Transfer Library. A library providing         |

|                   | standardized point-to-point data transfer primitives   |

|                   | (register, transfer, notify) for inference engines,    |

|                   | abstracting underlying transports (intra-node          |

|                   | interconnect, RDMA, PCIe, and storage interfaces).     |

|                   | Multiple open-source and vendor implementations exist. |

+-------------------+--------------------------------------------------------+

| GIN               | GPU-Initiated Networking. A communication paradigm     |

|                   | where GPU threads directly initiate network operations |

|                   | (RDMA sends, one-sided puts) without CPU involvement,  |

|                   | reducing latency by eliminating CPU-GPU                |

|                   | synchronization.                                       |

+-------------------+--------------------------------------------------------+

| PagedAttention    | A memory management technique for KV caches that       |

|                   | stores attention keys and values in fixed-size pages   |

|                   | (typically, 16-64 KB), enabling non-contiguous         |

|                   | allocation and reducing memory fragmentation.          |

+-------------------+--------------------------------------------------------+

| Continuous        | A scheduling technique that dynamically adds new       |

| Batching          | requests to an active inference batch as decode slots  |

|                   | become available, improving GPU utilization compared   |

|                   | to static batching.                                    |

+-------------------+--------------------------------------------------------+

| Prefix Caching    | Reuse of previously computed KV cache segments for     |

|                   | prompts that share a common prefix (e.g., system       |

|                   | prompt), avoiding redundant prefill computation.       |

+-------------------+--------------------------------------------------------+

| DUT               | Device Under Test. In this document, the DUT is one or |

|                   | more network fabric elements (switches, NICs, or the   |

|                   | complete fabric) whose performance impact on inference  |

|                   | serving is being characterized.                        |

+-------------------+--------------------------------------------------------+

| SUT               | System Under Test. The complete inference serving      |

|                   | system including accelerators, NICs, fabric, and       |

|                   | serving software, when end-to-end metrics are being    |

|                   | measured.                                              |

+-------------------+--------------------------------------------------------+

| RT                | Router Tester / Traffic Generator. Test equipment      |

|                   | capable of generating and receiving network traffic at |

|                   | specified rates with timestamping accuracy sufficient  |

|                   | for the measurements defined herein.                   |

+-------------------+--------------------------------------------------------+

3. Test Topology and Architecture

3.1. Reference Fabric Topologies

The reference topologies from the companion training document (2-Tier Clos, 3-Tier Clos, Rail-Optimized) remain applicable.

Inference serving introduces additional topology considerations related to disaggregated prefill/decode placement and MoE expert distribution.

3.1.1. Topology A: 2-Tier Clos (Leaf-Spine)

Applicable to inference clusters up to approximately 2,048 accelerators. Prefill and decode worker groups SHOULD be placed on separate leaf switches (or separate leaf switch groups) to isolate KV cache transfer traffic from decode-to-client response traffic. Expert parallelism (EP) traffic within a single MoE dispatch group SHOULD be confined to a single leaf switch or a minimal number of leaf switches to minimize spine-hop latency.

3.1.2. Topology B: 3-Tier Clos (Leaf-Spine-Superspine)

Required for inference clusters exceeding 2,048 accelerators or for multi-model serving deployments where different model instances occupy different fabric pods. KV cache transfer traffic between prefill and decode workers in different pods traverses the superspine tier, making superspine bandwidth and latency critical.

3.1.3. Topology C: Disaggregated Prefill/Decode Placement

A topology variant specific to inference serving in which prefill workers and decode workers are placed in distinct physical locations within the fabric, connected by a dedicated KV cache transfer network

segment. This topology enables independent scaling of prefill and decode resources and allows heterogeneous hardware (e.g., high-compute GPUs for prefill, high-memory-bandwidth GPUs for decode).

+------------------------+

|    Request Router      |

|    (KV-Aware LB)       |

+----------+-------------+

|

+----------------+----------------+

|                                 |

+---------v---------+             +---------v---------+

|   Prefill Pool    |             |   Decode Pool     |

|   (xP workers)    |             |   (yD workers)    |

|   High Compute    |             |   High Mem BW     |

|   TP=8, DP=N/8    |             |   TP=8, DP=M/8    |

+--------+----------+             +----------+--------+

|                                   |

|     KV Cache RDMA Transfer        |

|     (One-sided PUT / Signal)      |

+-----------------------------------+

Figure 1: Disaggregated Prefill/Decode Inference Topology

3.2. Disaggregated Prefill/Decode Topology

The disaggregated topology separates the inference pipeline into physically distinct pools connected by the fabric. The test topology MUST include the following components:

Prefill Worker Pool: N_P nodes, each containing G accelerators with high-compute capability. These workers execute the prefill phase and generate KV cache state. Tensor Parallelism (TP) is applied within each node; Data Parallelism (DP) is applied across nodes. Each prefill worker communicates with one or more decode workers via RDMA-based KV cache transfer. Decode Worker Pool: N_D nodes, each containing G accelerators with high memory bandwidth. These workers receive KV cache state from prefill workers and execute the autoregressive decode phase. DP Attention may partition the KV cache across DP ranks within the decode pool, requiring AllToAll communication during decode. KV Cache Transfer Network: The fabric segment connecting prefill and decode pools. This segment carries one-sided RDMA PUT operations (or PUT-with-signal) transferring KV cache blocks from prefill GPU memory to decode GPU memory. The transfer path may traverse NVLink (intra-node), PCIe/CXL (to NIC), and Ethernet/InfiniBand (inter-node).

Request Router: A network-layer or application-layer loa  balancer that assigns incoming inference requests to prefill workers and subsequently routes KV cache to the appropriate decode workers. KV-aware routing and prefix-aware caching policies are under test.

3.3. Device Under Test (DUT) Identification

The following table defines the DUT configurations tested in this document. Each DUT configuration represents a different level of fabric abstraction.

+--------+--------------------+--------------------------------------------------+
| DUT ID | Description        | Components Under Test                            |
+--------+--------------------+--------------------------------------------------+
| DUT-S  | Single Switch      | Individual leaf or spine switch forwarding       |
|        |                    | inference traffic. Measures per-hop latency,     |
|        |                    | buffer absorption, ECN marking accuracy.         |
+--------+--------------------+--------------------------------------------------+
| DUT-F  | Complete Fabric    | End-to-end fabric from prefill NIC egress to     |
|        |                    | decode NIC ingress. Measures fabric-level KV     |
|        |                    | cache transfer latency, throughput, and          |
|        |                    | congestion behavior.                             |
+--------+--------------------+--------------------------------------------------+
| DUT-N  | NIC Transport      | NIC RDMA transport stack processing KV cache     |
|        |                    | transfer operations. Measures RDMA verb          |
|        |                    | completion latency, one-sided PUT bandwidth,     |
|        |                    | QP scaling.                                      |
+--------+--------------------+--------------------------------------------------+
| DUT-PD | Prefill-Decode     | The complete data path from prefill GPU memory   |
|        | Path               | through NIC, fabric, NIC, to decode GPU memory.  |
|        |                    | Measures end-to-end KV cache transfer including  |
|        |                    | NVLink, PCIe, and fabric segments.               |
+--------+--------------------+--------------------------------------------------+
| SUT-E  | End-to-End System  | Complete inference serving system including      |
|        |                    | inference serving software, RDMA transfer        |
|        |                    | libraries, fabric, and accelerators. Measures    |
|        |                    | TTFT, ITL, TPS as functions of fabric            |
|        |                    | performance.                                     |
+--------+--------------------+--------------------------------------------------+

3.4. Traffic Generator and Workload Emulator Requirements

Tests in this document require one or both of the following traffic generation modes. The mode used MUST be documented in all test reports.

3.4.1. Hardware Traffic Generator (RT) — Minimum Requirements

The hardware traffic generator MUST satisfy all of the following:

(a) RDMA traffic generation supporting RoCEv2 and, where tested, UET transport; configurable RDMA verb types (one-sided PUT, PUT-with-signal, two-sided SEND/RECV).

(b) Configurable message sizes from 4 KB (minimum KV cache page) to 256 MB (large KV cache block).

(c) Configurable QP counts from 1 QP to a minimum of 256 QPs per source-destination port pair.

3.4.2. Software Workload Emulator (WE) — Minimum Requirements

A software workload emulator runs on actual accelerators and generates realistic inference workloads. The WE MUST support all of the following:

(a) Configurable prompt length distributions: uniform, Zipf, and trace-replay modes.

(b) Configurable output length distributions and configurable request arrival rates: Poisson, bursty, and trace-replay.

(c) Disaggregated prefill/decode execution with actual RDMA-based KV cache transfer between prefill and decode worker pools.

(d) MoE expert parallelism with actual AllToAll dispatch where MoE-specific tests (Section 7) are performed.

(e) Measurement instrumentation providing per-request TTFT and ITL with timestamp accuracy <= 1 millisecond.

When a software workload emulator is used, the complete software configuration MUST be documented per Section 3.3, as framework version, RDMA library version, and GPU driver version materially affect results.

4. KPI Framework and Metrics Taxonomy

This section defines the Key Performance Indicators measured across all test categories. KPIs are organized into four tiers:

Primary Latency KPIs (end-user-facing response time metrics), Primary Throughput KPIs (system-level capacity metrics), Fabric-Level KPIs (network-specific  measurements), and Fabric Health Indicators (operational monitoring metrics).

NOTE: Where numerical reference values appear in the Target column of the KPI tables below (including TTFT, ITL, and other latency targets), these values are provided as informational reference points reflecting current industry observations for interactive inference workloads. They do NOT constitute benchmarking acceptance criteria or performance requirements. Per the BMWG charter, the definition of acceptance criteria or performance requirements is explicitly outside the scope of this Working Group. SLO values cited in this document are

illustrative examples from current industry practice. Implementers MAY use these values as contextual references when interpreting results; they MUST NOT be used as pass/fail thresholds in vendor evaluations. Deployment-specific SLOs will vary by application,

model architecture, and operator requirements.

NOTE: Target values in this section are NON-NORMATIVE illustrative reference points derived from current industry practice as of 2025-2026.

They do NOT constitute benchmarking acceptance criteria or performance requirements. Per BMWG charter, defining acceptance criteria is explicitly out of scope. Implementers MAY use these values as contextual references

only; they MUST NOT be used as pass/fail thresholds.

4.1. Primary Latency KPIs

+---------------+------+-----------------------------+-----------------+----------------------+

| KPI           | Unit | Definition                  | Target          | Measurement          |

|               |      |                             | (Interactive)   | Point                |

+---------------+------+-----------------------------+-----------------+----------------------+

| TTFT          | ms   | Time from request arrival   | < 500 ms P99    | SUT-E request/       |

|               |      | to first output token       |                 | response boundary    |

|               |      | emission                    |                 |                      |

+---------------+------+-----------------------------+-----------------+----------------------+

| ITL           | ms   | Time between successive     | < 50 ms P99     | SUT-E token emission |

|               |      | output tokens               |                 | timestamps           |

+---------------+------+-----------------------------+-----------------+----------------------+

| TTFT_fabric   | ms   | Fabric contribution to      | < 300 ms P99    | DUT-PD NIC-to-NIC    |

|               |      | TTFT (KV cache transfer     |                 | measurement          |

|               |      | latency)                    |                 |                      |

+---------------+------+-----------------------------+-----------------+----------------------+

| ITL_fabric    | ms   | Fabric contribution to      | < 5 ms P99      | DUT-F EP dispatch    |

|               |      | ITL (EP dispatch latency    |                 | round-trip           |

|               |      | per decode step)            |                 |                      |

+---------------+------+-----------------------------+-----------------+----------------------+

| E2E_latency   | ms   | End-to-end request latency  | Varies by       | SUT-E request/       |

|               |      | from arrival to completion  | output length   | response boundary    |

|               |      | of all output tokens        |                 |                      |

+---------------+------+-----------------------------+-----------------+----------------------+

4.2. Primary Throughput KPIs

+---------------+---------------+-----------------------------+----------------------+
| KPI           | Unit          | Definition                  | Measurement Point    |
+---------------+---------------+-----------------------------+----------------------+
| TPS_input     | tokens/s      | Aggregate input (prefill)   | SUT-E prefill        |
|               |               | tokens processed per second | completion events    |
|               |               | across all workers          |                      |
+---------------+---------------+-----------------------------+----------------------+
| TPS_output    | tokens/s      | Aggregate output (decode)   | SUT-E token emission ||               |               | tokens generated per second | events               |
|               |               | across all workers          |                      |
+---------------+---------------+-----------------------------+----------------------+
| TPS_per_GPU   | tokens/s/GPU  | Output tokens per second    | SUT-E per-worker     |
|               |               | normalized by number of     | counters             |
|               |               | decode GPUs                 |                      |
+---------------+---------------+-----------------------------+----------------------+
| Goodput       | tokens/s      | Effective output token rate | SUT-E successful     |
|               |               | excluding tokens from       | completion events    |
|               |               | preempted, evicted, or      |                      |
|               |               | failed requests             |                      |
+---------------+---------------+-----------------------------+----------------------+
| KV_BW         | GB/s          | Aggregate KV cache transfer | DUT-PD RDMA counters |
|               |               | bandwidth between prefill   |                      |
|               |               | and decode pools            |                      |
+---------------+---------------+-----------------------------+----------------------+
| Request_Rate  | req/s         | Maximum sustained request   | SUT-E admission      |
|               |               | arrival rate meeting all    | control boundary     |
|               |               | latency SLOs                |                      |
+---------------+---------------+-----------------------------+----------------------+

4.3. Fabric-Level KPIs

+------------------------+-------+--------------------------------+-------+
| KPI                    | Unit  | Definition                     | DUT   |
+------------------------+-------+--------------------------------+-------+
| KV_xfer_latency        | us    | One-sided RDMA PUT completion  | DUT-N |
|                        |       | time for a single KV cache     |       |
|                        |       | block transfer                 |       |
+------------------------+-------+--------------------------------+-------+
| KV_xfer_bandwidth      | GB/s  | Sustained unidirectional KV    | DUT-N |
|                        |       | cache transfer throughput per  |       |
|                        |       | NIC port                       |       |
+------------------------+-------+--------------------------------+-------+
| EP_alltoall_latency    | us    | Round-trip time for a complete | DUT-F |
|                        |       | MoE expert parallelism         |       |
|                        |       | AllToAll dispatch              |       |
+------------------------+-------+--------------------------------+-------+
| EP_alltoall_bandwidth  | GB/s  | Aggregate AllToAll bandwidth   | DUT-F |
|                        |       | across all EP ranks during     |       |
|                        |       | dispatch                       |       |
+------------------------+-------+--------------------------------+-------+
| Fabric_FCT             | us    | Flow completion time for a KV  | DUT-F |
|                        |       | cache transfer flow through    |       |
|                        |       | the fabric                     |       |
+------------------------+-------+--------------------------------+-------+
| Buffer_utilization     | %     | Peak switch buffer utilization | DUT-S |
|                        |       | during KV cache transfer       |       |
|                        |       | bursts                         |       |
+------------------------+-------+--------------------------------+-------+
| ECN_marking_rate       | %     | Fraction of packets marked     | DUT-S |
|                        |       | with ECN-CE during inference   |       |
|                        |       | traffic                        |       |
+------------------------+-------+--------------------------------+-------+
| PFC_frame_count        | frames| Number of PFC PAUSE frames     | DUT-S |
|                        |       | generated per unit time        |       |
+------------------------+-------+--------------------------------+-------+
| Link_utilization       | %     | Average and peak link          | DUT-F |
|                        |       | utilization on fabric links    |       |
|                        |       | carrying inference traffic     |       |
+------------------------+-------+--------------------------------+-------+
| Packet_drop_rate       | ppm   | Packets dropped per million    | DUT-F |
|                        |       | due to buffer overflow or      |       |
|                        |       | transport error                |       |
+------------------------+-------+--------------------------------+-------+

4.4. Fabric Health Indicators

+------------------+------------------+---------------------------------------+

| Indicator        | Threshold        | Description                           |

+------------------+------------------+---------------------------------------+

| CPU Utilization  | < 30%            | Control plane CPU usage on switches   |

| (switch)         |                  | under inference traffic load          |

+------------------+------------------+---------------------------------------+

| Memory Usage     | < 70%            | TCAM, buffer, and control plane       |

| (switch)         |                  | memory usage                          |

+------------------+------------------+---------------------------------------+

| FEC Error Rate   | < 1e-12          | Forward Error Correction              |

|                  | post-FEC BER     | effectiveness on fabric links         |

+------------------+------------------+---------------------------------------+

| CRC Error Count  | 0                | Layer 2 CRC errors on any fabric link |

+------------------+------------------+---------------------------------------+

| BGP/OSPF         | 0 flaps          | Routing protocol adjacency stability  |

| Stability        |                  | under inference load                  |

+------------------+------------------+---------------------------------------+

| NIC QP State     | 100% active      | All RDMA Queue Pairs in active state  |

|                  |                  | (no error/reset)                      |

+------------------+------------------+---------------------------------------+

| GPU-NIC PCIe BW  | > 90% of         | PCIe Gen5 x16 bandwidth utilization   |

|                  | theoretical      | between GPU and NIC                   |

+------------------+------------------+---------------------------------------+

5. Test Category 1: RDMA KV Cache Transfer Benchmarks

KV cache transfer between disaggregated prefill and decode workers is the defining fabric workload for inference serving. Unlike training collectives (AllReduce, AllGather) which are periodic and predictable, KV cache transfers are event-driven (triggered by prefill completion) and bursty. This section defines benchmarks for characterizing fabric performance under KV cache transfer workloads.

5.1. Point-to-Point KV Cache Transfer Throughput

Objective: To determine the maximum sustained KV cache transfer throughput between a single prefill worker NIC and a single decode worker NIC across the DUT fabric.

Procedure: Configure a single RDMA connection (QP) between the prefill and decode endpoints. Send a sequence of one-sided RDMA PUT operations with message sizes corresponding to KV cache block sizes. The message size sequence MUST include: 64 KB (single attention page), 256 KB, 1 MB, 4 MB, 16 MB, 64 MB, 256 MB (large prompt KV cache), and 1 GB. For each message size, transmit at the maximum rate sustainable by the NIC for a minimum of 60 seconds per trial. Record the achieved throughput in GB/s.

Repeat for 1, 4, 8, 16, 32, 64, and 128 concurrent QPs. The DUT is the fabric path from NIC to NIC.

Measurement: Record throughput (GB/s), CPU utilization on both endpoints, GPU memory-to-NIC transfer overhead, and NIC hardware offload utilization. The test MUST be repeated a minimum of 20 times per configuration and the average reported.

Reporting Format: Results SHOULD be reported as a multi-line graph with message size (log scale) on the X axis and throughput (GB/s) on the Y axis. Separate lines for each QP count. A reference line showing theoretical NIC line rate MUST be included.

5.2. KV Cache Transfer Latency

Objective: To determine the latency of individual KV cache block transfers across the DUT fabric under varying load conditions.

Procedure: Using the same endpoint configuration as Test 5.1, measure the completion time of individual RDMA PUT operations. Latency is measured from the initiation of the PUT verb on the prefill NIC to receipt of the completion signal on the decode NIC (for PUT-with-signal) or to polling of the remote completion queue. Measure latency under unloaded conditions (single outstanding operation) and under loaded conditions (background traffic at 25%, 50%, 75%, and 90% of fabric capacity). Message sizes MUST include 64 KB, 1 MB, 16 MB, and 256 MB.

Measurement: Report latency at P50, P95, P99, and P99.9 percentiles. The test MUST be repeated a minimum of 20 trials of at least 120 seconds each per configuration. The difference between P99 and P50 (tail latency spread) SHOULD be reported as a derived metric.

Reporting Format: Results SHOULD be reported as a table with columns for message size, background load level, and latency at each percentile. A complementary CDF plot of latency distribution for selected configurations SHOULD be included.

5.3. Concurrent KV Cache Transfer Scaling

Objective: To characterize how aggregate KV cache transfer performance scales as the number of concurrent prefill-to-decode transfer pairs increases.

Procedure: Configure N concurrent prefill-decode endpoint pairs, where N ranges from 1 to the maximum supported by the fabric (e.g., 1, 2, 4, 8, 16, 32, 64, 128 pairs). Each pair executes continuous KV cache transfers of 16 MB messages (representative of a medium-length prompt). Measure aggregate throughput and per-pair latency as N increases. The DUT is the complete fabric.

Measurement: Report aggregate throughput (GB/s), per-pair median latency (us), per-pair P99 latency (us), Jain Fairness Index across pairs, and maximum fabric link utilization observed. The test MUST be repeated a minimum of 20 times per value of N.

Reporting Format: Results SHOULD be reported as a dual-axis graph with N (concurrent pairs) on the X axis, aggregate throughput on the left Y axis, and P99 latency on the right Y axis. The JFI value for each N SHOULD be annotated.

5.4. Multi-Tier Storage Transfer Characterization

Objective: To characterize KV cache transfer performance across the memory/storage hierarchy: GPU HBM to GPU HBM (inter-node RDMA), GPU HBM to remote CPU DRAM (offload), CPU DRAM to GPU HBM (reload), and GPU HBM to NVMe/SSD (persistent cache).

Procedure: For each tier pair, measure unidirectional transfer throughput and latency for message sizes of 1 MB, 16 MB, and 256 MB. Use zero-copy transfers where supported (GDS for NVMe, GPUDirect RDMA for inter-node). The DUT encompasses the complete transfer path including PCIe, NVLink, and fabric segments.

Measurement: Report throughput (GB/s) and latency (P50, P99) for each tier pair and message size. Report the tier throughput ratio relative to GPU-to-GPU RDMA as a derived metric.

Reporting Format: Results SHOULD be reported as a table with rows for each tier pair and columns for throughput and latency at each message size.

6. Test Category 2: Prefill/Decode Disaggregation Benchmarks

Disaggregated prefill/decode serving separates the two phases onto distinct hardware pools to enable independent optimization and scaling.

This section benchmarks the fabric's ability to support the resulting KV cache transfer traffic patterns and their impact on end-to-end inference metrics.

6.1. End-to-End Disaggregated TTFT

Objective: To measure TTFT as a function of prompt length in a disaggregated serving configuration, isolating the fabric contribution.

Procedure: Configure a disaggregated serving system (SUT-E) with a specified xPyD ratio (e.g., 3P9D for a 12-node cluster). Submit inference requests with prompt lengths of 128, 512, 1024, 2048, 4096, 8192, and 16384 tokens. For each prompt length, measure the total TTFT and decompose it into: T_prefill (prefill compute time), T_transfer (KV cache fabric transfer time, measured at DUT-PD), and T_decode_init (first decode step time). The fabric contribution (T_transfer) is the primary measurement of interest.

Measurement: Report TTFT (ms) and its decomposition at P50, P95, and P99 percentiles. The ratio T_transfer/TTFT (fabric fraction) SHOULD be reported as a derived metric. The test MUST be repeated a minimum of 20 trials per prompt length.

Reporting Format: Results SHOULD be reported as a stacked bar chart with prompt length on the X axis and TTFT (ms) on the Y axis, with bars decomposed into T_prefill, T_transfer, and T_decode_init. A table of numerical values MUST accompany the chart.

6.2. xPyD Ratio Optimization

Objective: To determine the optimal, prefill-to-decode resource ratio for a given model, prompt distribution, and latency SLO, as limited by fabric transfer capacity.

Procedure: For a fixed total number of nodes N (e.g., 12), iterate over xPyD ratios: 1P11D, 2P10D, 3P9D, 4P8D, 6P6D, 8P4D, 10P2D, 11P1D. For each ratio, submit a sustained request stream matching a target request rate (e.g., 10, 50, 100, 200 req/s) with a specified prompt length distribution (e.g., Zipf with alpha=1.0 over [128, 8192] tokens). Measure TTFT P99, ITL P99, TPS_output, and Goodput for each configuration.

Measurement: Report all four metrics for each xPyD ratio and request rate. Identify the Pareto-optimal ratio(s) that maximize TPS_output while meeting TTFT P99 \< 500 ms and ITL P99 \< 50 ms. Reporting Format: Results SHOULD be reported as a multi-panel figure with one panel per request rate, each showing xPyD ratio on the X axis and metrics on dual Y axes (TTFT/ITL on left, TPS on right). The Pareto frontier SHOULD be highlighted.

6.3. Heterogeneous Parallelism Configuration

Objective: To evaluate the fabric impact of using different parallelism strategies on prefill vs. decode pools in a disaggregated configuration.

Procedure: Test the following parallelism configurations in a disaggregated cluster: (a) Prefill TP=8, Decode TP=8 (baseline, same parallelism), (b) Prefill TP=8, Decode TP=4 with DP_Attention=2 (reduced TP, added DP), (c) Prefill TP=4 with DP=2, Decode TP=2 with DP_Attention=4 (aggressive DP). For each configuration, measure KV cache transfer patterns (number of flows, message sizes, concurrency), fabric bandwidth consumed, and end-to-end TTFT/ITL.

Measurement: Report the number of concurrent RDMA flows, aggregate bandwidth (GB/s), TTFT (ms), and ITL (ms) at P50 and P99 for each configuration.

6.4. Prefill Queue Depth Impact on Transfer Latency

Objective: To measure how queuing of prefill requests (due to compute contention) affects KV cache transfer burstiness and fabric congestion.

Procedure: Oversubscribe the prefill pool by submitting requests at a rate exceeding prefill capacity. Measure the resulting KV cache transfer burst characteristics: burst size (number of concurrent transfers), burst duration, inter-burst gap, and peak fabric bandwidth demand. Vary the oversubscription ratio from 1.0x (saturated) to 2.0x in 0.25x increments.

Measurement: Report burst size distribution, peak and average fabric bandwidth, KV transfer latency P99, and ECN/PFC event counts as functions of oversubscription ratio.

7. Test Category 3: MoE Expert Parallelism Benchmarks

Mixture-of-Experts models distribute expert sub-networks across GPUs and route tokens to the appropriate experts via AllToAll communication.

This section benchmarks the fabric's ability to support the resulting fine-grained, latency-sensitive inter-GPU traffic patterns.

7.1. AllToAll Dispatch Throughput

Objective: To determine the maximum AllToAll dispatch throughput for MoE expert parallelism across the DUT fabric. Procedure: Configure N GPUs in an EP group (e.g., N = 8, 16, 32, 64, 96). Each GPU holds M/N experts where M is the total expert count.

Generate a synthetic MoE dispatch workload where each GPU sends token embeddings to the experts selected by a top-k routing function (k=2 typical). The dispatch payload per GPU per MoE layer is: (batch_size * seq_len * top_k * hidden_dim * precision_bytes) / N. Measure the aggregate AllToAll bandwidth and per-dispatch latency for batch sizes of 1, 8, 32, 128, and 512 tokens, and EP group sizes of 8, 16, 32, 64, and 96.

Measurement: Report aggregate bandwidth (GB/s), per-dispatch latency (us) at P50 and P99, and GPU idle time waiting for dispatch completion. The test MUST be repeated a minimum of 20 times per configuration.

Reporting Format: Results SHOULD be reported as a heatmap with EP group size on the Y axis, batch size on the X axis, and throughput (GB/s) as the color dimension. A companion latency table MUST be included.

7.2. Normal vs. Low-Latency Dispatch Mode Comparison

Objective: To compare fabric performance under the two dispatch modes defined by the MoE dispatch library in use: Normal Dispatch (optimized for prefill, dynamic shapes, incompatible with CUDA Graph) and Low-Latency Dispatch (optimized for decode, fixed shapes, CUDA Graph compatible). Procedure: Execute identical MoE dispatch workloads in both modes for EP group sizes of 8, 32, and 96 GPUs. For Normal Dispatch, use batch sizes of 128, 256, and 512 (typical prefill). For Low-Latency Dispatch, use batch sizes of 1, 4, 8, and 16 (typical decode). Measure dispatch latency, fabric bandwidth, and the impact of CUDA Graph on kernel launch overhead.

Measurement: Report per-dispatch latency (us) at P50, P95, P99 for each mode. Report the latency ratio LL/Normal for equivalent EP sizes.

Report the fabric bandwidth utilization (%) for each mode.

7.3. Wide Expert Parallelism Scaling

Objective: To characterize AllToAll dispatch performance as EP group size scales beyond a single node (wide EP), requiring inter-node fabric communication.

Procedure: Scale the EP group from intra-node only (EP=8 on a single 8-GPU node) to wide EP (EP=16, 32, 48, 64, 96 spanning 2, 4, 6, 8, 12 nodes). For each EP size, measure: intra-node dispatch bandwidth (NVLink), inter-node dispatch bandwidth (RDMA/fabric), total dispatch latency, and the ratio of inter-node to intra-node bandwidth. Use a fixed batch size of 128 tokens and a representative MoE model configuration (256 experts, top-2 routing).

Measurement: Report total dispatch latency (us), inter-node bandwidth (GB/s), and latency decomposition (intra-node vs. inter-node fraction).

Report the scaling efficiency: (EP=8 latency) / (EP=N latency) * (N/8).

7.4. Expert Parallelism and KV Cache Transfer Contention

Objective: To measure the mutual interference between EP AllToAll dispatch traffic and KV cache transfer traffic when both share the same fabric links.

Procedure: On a shared fabric, simultaneously execute: (a) continuous KV cache transfers at a sustained rate (e.g., 50%, 75% of fabric capacity), and (b) periodic EP AllToAll dispatches (one per MoE layer forward pass). Measure the impact of each traffic class on the other: KV cache transfer latency with and without concurrent EP traffic, EP dispatch latency with and without concurrent KV cache traffic, and the aggregate fabric utilization.

Measurement: Report KV_xfer_latency P99 (us) and EP_alltoall_latency P99 (us) for the isolated and contended cases.

Report the contention penalty as the ratio of contended P99 to isolated P99 for each traffic class.

Report ECN/PFC event counts during contention.

8. Test Category 4: Congestion Management Benchmarks

Inference traffic patterns differ from training in their burstiness, heterogeneity (mixed KV cache transfers and EP dispatches), and latency sensitivity. This section adapts the congestion management benchmarks

from the companion training document to inference-specific traffic profiles.

8.1. ECN Marking Under Inference Incast

Objective: To verify that ECN marking thresholds are correctly applied when multiple prefill workers simultaneously transfer KV cache blocks to a single decode worker (incast pattern). Procedure: Configure M prefill workers (M = 2, 4, 8, 16, 32) to simultaneously transfer 16 MB KV cache blocks to a single decode worker port.

Measure the ECN-CE marking rate at the leaf switch connected to the decode worker. Repeat for ECN marking thresholds of 100 KB, 500 KB, 1 MB, and 5 MB. The DUT is the individual leaf switch (DUT-S).

Measurement: Report the ECN marking rate (fraction of marked packets), the onset of marking (time from first transfer to first ECN mark), queue depth at marking onset, and aggregate throughput achieved. Repeat a minimum of 20 times per configuration.

8.2. PFC Behavior Under Bursty KV Cache Transfers

Objective: To characterize PFC PAUSE frame generation and propagation under bursty KV cache transfer patterns typical of disaggregated serving.

Procedure: Generate KV cache transfer bursts matching the following profile: N_burst concurrent transfers (N_burst = 4, 8, 16, 32), each of size S_block (16 MB), arriving within a window of T_arrival (100 us, 1 ms, 10 ms). Vary the PFC threshold from 10 KB to 1 MB. Measure the number of PFC PAUSE frames generated, PAUSE duration, impact on non-KV-cache traffic sharing the same port, and time to resume normal forwarding after burst completion.

Measurement: Report PFC frame count, total PAUSE duration (us), head-of-line blocking delay imposed on other traffic classes (us), and KV cache transfer completion time.

8.3. Congestion Control Convergence for Mixed Traffic

Objective: To measure the convergence time of DCQCN (or UET congestion control) when KV cache transfer traffic and EP AllToAll dispatch traffic share fabric capacity.

Procedure: Establish a sustained KV cache transfer at 80% of fabric capacity. Introduce EP AllToAll dispatch traffic on the same fabric links. Measure the time for the congestion control algorithm to converge to a stable rate allocation between the two traffic classes. Repeat the

test with the roles reversed (EP traffic established first, KV cache introduced).

Measurement: Report convergence time (ms) to within 5% of steady-state rates, steady-state bandwidth allocation between traffic classes, packet loss during convergence, and Jain Fairness Index of the steady-state allocation.

8.4. PFC Storm and Deadlock Resilience

Objective: To verify that the fabric does not enter a PFC storm or deadlock condition under adversarial inference traffic patterns.

Procedure: Per the companion training document (Section 7.4), generate a PFC storm scenario by creating circular buffer dependency across multiple switches. Simultaneously inject KV cache transfer traffic on all affected paths. Monitor for: PFC storm propagation (PAUSE frames cascading beyond the congestion point), deadlock (zero throughput on any link for \> 100 ms), and recovery time after the adversarial condition is removed. The test duration MUST be at least 300 seconds.

Measurement: Report whether PFC storm occurred (yes/no), deadlock occurred (yes/no), maximum PAUSE propagation depth (number of hops), maximum zero-throughput duration (ms), and recovery time (ms).

9. Test Category 5: Request Routing and Load Balancing

Inference serving introduces application-layer routing decisions (which prefill worker processes a request, which decode worker receives the KV cache) that interact with fabric-layer load balancing (ECMP, flowlet, packet spray). This section benchmarks both layers.

9.1. KV-Aware Request Routing Efficacy

Objective: To measure the effectiveness of KV-aware request routing, where the request router considers decode worker KV cache memory occupancy and fabric path congestion when assigning requests.

Procedure: Configure a request router with KV-aware routing enabled (e.g., an application-layer KV-aware request router or intelligent scheduler).

Submit a sustained request stream at rates of 10, 50, 100, and 200 req/s with varying prompt lengths. Measure decodes worker GPU memory utilization variance, KV cache transfer latency variance, TTFT variance, and the frequency of KV cache evictions due to memory pressure. Compare against round-robin routing (baseline).

Measurement: Report the coefficient of variation (CV) of decode worker memory utilization, P99 TTFT, P99 ITL, KV cache eviction rate, and Goodput for both KV-aware and round-robin routing.

9.2. Prefix-Aware Cache Hit Rate

Objective: To measure the fabric bandwidth savings achieved by prefix-aware caching, where requests with common prefixes (e.g., system prompt) are routed to workers that already hold the corresponding KV cache segment.

Procedure: Generate a request workload where P% of requests share a common prefix of L tokens (P = 25%, 50%, 75%, 90%; L = 256, 512, 1024, 2048). Measure the prefix cache hit rate, the resulting reduction in KV cache transfer bandwidth, and the impact on TTFT. Compare against non-prefix-aware routing.

Measurement: Report cache hit rate (%), fabric bandwidth reduction (%), TTFT reduction (ms), and TPS improvement (%) for each (P, L) combination.

9.3. ECMP and Dynamic Load Balancing Under Inference Traffic

Objective: To evaluate fabric-layer load balancing (ECMP, flowlet-based DLB, packet spray) effectiveness under inference traffic patterns characterized by a mix of large KV cache flows and small EP dispatch flows.

Procedure: Per the companion training document (Section 8), measure link utilization uniformity under: (a) KV cache transfers only (large flows, 16 MB+), (b) EP AllToAll dispatches only (small flows, \< 1 MB), (c) mixed KV cache and EP traffic. For each scenario, measure the Jain Fairness Index of link utilization across parallel fabric paths and the maximum link oversubscription ratio.

Measurement: Report JFI, maximum link utilization (%), minimum link utilization (%), and the oversubscription ratio for each scenario and load balancing algorithm.

9.4. Jain Fairness Index for Decode Worker Utilization

Objective: To measure how evenly the fabric distributes KV cache transfer load across decode workers, which directly impacts decode worker GPU utilization balance.

Procedure: With N_D decode workers (N_D = 8, 16, 32, 64), submit a sustained request stream and measure the per-worker KV cache receive rate (GB/s), per-worker GPU utilization (%), and per-worker output TPS.

Calculate the Jain Fairness Index across decode workers for each metric.

Measurement: Report JFI for KV cache receive rate, GPU utilization, and output TPS. Report the max/min ratio for each metric.

10. Test Category 6: Latency Benchmarks

Inference latency is the primary user-facing quality metric. This section defines benchmarks that isolate the fabric's contribution to end-to-end inference latency under various operating conditions.

10.1. TTFT Under Varying Prompt Lengths

Objective: To characterize TTFT as a function of prompt length, isolating the fabric-dependent KV cache transfer component.

Procedure: Using the SUT-E configuration defined in Section 6.1, submit single requests (no concurrent load) with prompt lengths of 128, 256, 512, 1024, 2048, 4096, 8192, and 16384 tokens. For each prompt length, measure TTFT and decompose into T_prefill, T_transfer, and T_decode_init. The KV cache size scales linearly with prompt length; the expected transfer size for a 70B model at FP16 is approximately 0.33 MB per token.

Measurement: Report TTFT, T_transfer, and T_transfer/TTFT at P50, P95, P99 for each prompt length. The test MUST be repeated a minimum of 100 times per prompt length to obtain stable percentile estimates.

Reporting Format: Results SHOULD be reported as a line graph with prompt length (tokens) on the X axis and TTFT (ms) on the Y axis, with separate lines for P50 and P99. The T_transfer component SHOULD be shown as a shaded region.

10.2. ITL Characterization and Tail Latency

Objective: To characterize inter-token latency distribution and identify fabric-induced tail latency during the decode phase. Procedure: Submit a single long-output request (e.g., 2048 output tokens) and record the timestamp of each emitted token. Compute ITL as the difference between consecutive token timestamps. Repeat under: (a) unloaded fabric (single request, no background traffic), (b) loaded fabric (background KV cache transfers at 50% of capacity), and (c) heavily loaded fabric (background traffic at 90% of capacity plus concurrent EP dispatches).

Measurement: Report ITL at P50, P95, P99, P99.9, and maximum for each load condition. Report the number of tokens exhibiting ITL \> 100 ms (stall events). The test MUST generate at least 10,000 ITL samples per condition.

10.3. End-to-End Latency Under Multi-Tenant Load

Objective: To measure inference latency when multiple models or model instances share the same fabric.

Procedure: Deploy two or more model instances (e.g., a 70B model and a 7B model, or two instances of the same model) on separate worker pools sharing the same fabric. Submit requests to both instances concurrently at specified rates. Measure TTFT and ITL for each instance independently.

Measurement: Report per-instance TTFT P99, ITL P99, and the interference penalty: (multi-tenant metric - single-tenant metric) / single-tenant metric * 100%.

10.4. Latency Sensitivity to Fabric Congestion

Objective: To establish the relationship between fabric congestion level and inference latency degradation.

Procedure: Inject controlled background traffic on the fabric at levels from 0% to 95% of capacity in 5% increments. At each level, submit inference requests at a fixed rate and measure TTFT and ITL. The background traffic SHOULD be non-inference traffic (e.g., storage I/O, model parameter loading) to simulate a shared fabric.

Measurement: Report TTFT P99 and ITL P99 as functions of background traffic level. Identify the inflection point (the traffic level at which latency begins to degrade significantly). Report the latency degradation factor at 50%, 75%, and 90% background load.

11. Test Category 7: Throughput Benchmarks

Inference throughput determines the cost-effectiveness of the serving deployment. This section benchmarks the fabric's impact on aggregate token generation throughput.

11.1. Aggregate Tokens Per Second

Objective: To determine the maximum sustained aggregate TPS achievable while meeting latency SLOs, as a function of fabric configuration.

Procedure: Using the SUT-E configuration, increase the request arrival rate from 1 req/s to the point where either TTFT P99 exceeds 500 ms or ITL P99 exceeds 50 ms. At each rate, measure TPS_output, TPS_input, Goodput, and all latency KPIs. The maximum rate at which all SLOs are met defines the SLO-bounded throughput.

Measurement: Report TPS_output, TPS_input, Goodput, TTFT P99, ITL P99, and fabric utilization at the SLO-bounded throughput.

Report the fabric utilization at the SLO boundary as a key efficiency metric.

11.2. Batch Size Scaling and Continuous Batching Impact

Objective: To measure the interaction between inference batch size, continuous batching, and fabric transfer patterns.

Procedure: Configure the serving system with varying maximum batch sizes (1, 4, 8, 16, 32, 64, 128). For each batch size, measure: (a) the number of concurrent KV cache transfers (batch size determines how many prefill completions trigger concurrent transfers), (b) aggregate fabric bandwidth consumed, (c) TPS_output, and (d) TTFT P99. Enable continuous batching and repeat. Measure the additional fabric traffic caused by dynamic batch slot reuse.

Measurement: Report TPS_output, TTFT P99, fabric bandwidth (GB/s), and peak concurrent transfers for each batch size, with and without continuous batching.

11.3. Goodput Under Preemption and Eviction

Objective: To measure the Goodput loss when fabric congestion forces KV cache eviction or request preemption.

Procedure: Oversubscribe the system beyond the SLO-bounded throughput (at 110%, 125%, 150%, and 200% of the rate found in Test 11.1). Measure the rate of KV cache evictions (requests whose KV cache is evicted from decode worker memory due to memory pressure, requiring re-prefill), request preemptions (requests interrupted to make room for higher-priority requests), and the resulting Goodput reduction.

Measurement: Report Goodput, eviction rate (evictions/s), preemption rate (preemptions/s), wasted fabric bandwidth (GB/s transferred for ultimately evicted KV caches), and the Goodput/TPS_output ratio (efficiency).

12. Test Category 8: Scale and Autoscaling

Inference serving clusters must scale dynamically to match request demand. This section benchmarks the fabric's behavior under scale-up, scale-down, and link failure conditions.

12.1. Fabric Scale Limits for Inference Clusters

Objective: To determine the maximum inference cluster size supportable by the DUT fabric while meeting performance requirements.

Procedure: Progressively scale the cluster from a minimal configuration (e.g., 2 nodes, 16 GPUs) to the fabric's capacity (e.g., 1024 nodes, 8192 GPUs). At each scale point, measure: (a) KV cache transfer throughput and latency, (b) EP AllToAll dispatch latency, (c) fabric control plane convergence time, (d) routing table size, and (e) end-to-end TTFT and TPS. The scaling sequence SHOULD follow powers of two: 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024 nodes.

Measurement: Report all KPIs at each scale point. Identify the scale limit as the point where any KPI degrades by more than 10% from the minimal-configuration baseline.

12.2. Dynamic Autoscaling Response Time

Objective: To measure the time required for the fabric to accommodate dynamic scaling of inference worker pools (adding/removing prefill or decode workers).

Procedure: Starting from a stable serving state, trigger a scale-up event (e.g., adding 4 decode nodes). Measure: (a) time from scale trigger to new nodes being reachable via the fabric (fabric convergence time), (b) time from fabric convergence to the first KV cache transfer landing on new nodes, (c) time to reach steady-state throughput including new nodes.

Repeat for scale-down events.

Measurement: Report fabric convergence time (ms), first-transfer time (ms), and time to steady-state (ms) for scale-up and scale-down events.

Report any packet loss or latency spikes during the scaling transition.

12.3. Link Failure Convergence Impact on Serving

Objective: To measure the impact of fabric link failures on inference serving performance and the convergence time to restore full service.

Procedure: During sustained inference serving at 80% of SLO-bounded

throughput, fail a single fabric link on: (a) a leaf-spine link carrying KV cache traffic, (b) a spine-spine link, (c) a link on the decode worker's leaf switch. Measure the duration of traffic disruption (packets lost, increased latency), time to routing convergence, and time to restore pre-failure TTFT and TPS levels. Repeat the test for dual link failures. Measurement: Report traffic disruption duration (ms), convergence time (ms), TTFT degradation during convergence (ms above baseline P99), TPS reduction during convergence (%), and time to full recovery (ms). The test MUST be repeated a minimum of 20 times per failure scenario.

13. Test Category 9: Soak and Stability

Long-running inference serving deployments must maintain performance without degradation over time. This section defines soak tests adapted for inference workloads.

13.1. 24-Hour Sustained Inference Load

Objective: To verify that the fabric maintains performance under continuous inference serving load for 24 hours.

Procedure: Configure the SUT-E at 80% of the SLO-bounded throughput determined in Test 11.1. Run a continuous request stream for 24 hours with a realistic prompt length distribution. Sample the following metrics every 15 minutes: TTFT P99, ITL P99, TPS_output, KV_xfer_latency P99, fabric link utilization, switch CPU/memory usage, NIC counters (RDMA retransmissions, QP errors), and PFC/ECN event counts.

Measurement: Report the trend of all sampled metrics over the 24-hour period. Flag any monotonic increase in latency or resource usage as a potential stability issue. There SHOULD be zero NIC QP errors, zero routing flaps, and less than 1% variation in TTFT P99 over the test duration.

13.2. KV Cache Memory Leak Detection

Objective: To detect memory leaks in the KV cache management subsystem that may manifest as fabric performance degradation over time.

Procedure: Monitor GPU memory, CPU memory, NIC registered memory regions, and RDMA memory region counts on all prefill and decode workers during the 24-hour soak test (Test 13.1). Record the number of active KV cache pages, the number of RDMA memory registrations, and the amount of pinned memory at each sampling interval.

Measurement: Report the trend of each monitored metric. Flag any monotonic increase as a potential leak. Report the maximum observed memory usage and the usage at the end of the 24-hour period.

13.3. Long-Running Serving Stability

Objective: To verify that fabric-dependent components (NIC RDMA transport, switch buffer management, routing protocol state) remain stable under continuous inference serving.

Procedure: During the 24-hour soak test, monitor: NIC QP state transitions (any transition from RTS to error state), switch buffer utilization trend, FEC error rate trend, BGP/OSPF adjacency stability, and RDMA retransmission rate. At the 12-hour mark, trigger a controlled perturbation (single link flap) and verify recovery.

Measurement: Report the count of any QP state transitions, maximum switch buffer utilization, FEC error trend, adjacency flap count, and RDMA retransmission count. Report the recovery time from the 12-hour link flap perturbation.

14. Reporting Format

All test results MUST be reported following the conventions established in RFC 2544 Section 26. In addition, the following inference-specific reporting requirements apply:

System Configuration Report: The report MUST include the complete system

configuration: model name and parameter count, parallelism strategy (TP, DP, EP, PP configuration for both prefills and decode pools), xPyD ratio, inference serving framework name and version, KV cache transfer library name and version, accelerator type and count, NIC type and firmware version, switch ASIC and software version, fabric topology, and link speeds.

Workload Characterization Report: The report MUST include the workload parameters: prompt length distribution (mean, P50, P99, distribution type), output length distribution, request arrival rate and distribution, number of concurrent requests, and prefix sharing percentage.

Results Reporting: For each test, results MUST include: the specific test identifier (e.g., Test 5.1), the DUT/SUT configuration tested, the number of trials, all measured KPI values with confidence intervals, and any anomalies observed.

Graphical reports SHOULD follow the formats specified in each test section.

+------------------------+-----------------------------+------------+
| Report Element         | Format                      | Required?  |
+------------------------+-----------------------------+------------+
| System Configuration   | Structured table per above  | MUST       |
+------------------------+-----------------------------+------------+
| Workload Parameters    | Structured table per above  | MUST       |
+------------------------+-----------------------------+------------+
| KPI Summary Table      | Table with all measured     | MUST       |
|                        | KPIs                        |            |
+------------------------+-----------------------------+------------+
| Latency Distribution   | CDF or histogram per        | SHOULD     |
| Plots                  | test section                |            |
+------------------------+-----------------------------+------------+
| Throughput vs. Scale   | Line chart per test section | SHOULD     |
| Graphs                 |                             |            |
+------------------------+-----------------------------+------------+
| Fabric Health          | Table per Section 4.4       | MUST       |
| Indicators             |                             |            |
+------------------------+-----------------------------+------------+
| Raw Data Appendix      | Machine-readable format     | MAY        |
|                        | (CSV, JSON)                 |            |
+------------------------+-----------------------------+------------+

15. Security Considerations

This document defines benchmarking methodologies for controlled laboratory testing. All tests MUST be conducted in isolated test environments that are not connected to production networks or the public Internet. The security considerations from RFC 2544 and RFC 6815 apply.

Additionally, implementers SHOULD be aware that RDMA-based KV cache transfer provides direct memory access between hosts; all RDMA connections in the test environment MUST use authenticated QPs where supported. The test results themselves may reveal performance characteristics that could inform denial-of-service attack vectors; results SHOULD be treated as sensitive when applicable.

16. IANA Considerations

This memo includes no request to IANA.

17. References

17.1. Normative References

[RFC2119] Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, DOI 10.17487 RFC2119, March 1997.

[RFC8174] Leiba, B., "Ambiguity of Uppercase vs Lowercase in RFC 2119 Key Words", BCP 14, RFC 8174, DOI 10.17487/RFC8174, May 2017.

[RFC1242] Bradner, S., "Benchmarking Terminology for Network Interconnect Devices", RFC 1242, DOI 10.17487/RFC1242, July 1991.

[RFC2544] Bradner, S. and J. McQuaid, "Benchmarking Methodology for Network Interconnect Devices", RFC 2544, DOI 10.17487/RFC2544, March 1999.

[RFC2889] Mandeville, R. and J. Perser, "Benchmarking Methodology for LAN Switching Devices", RFC 2889, DOI 10.17487/RFC2889, August 2000.

[RFC6349] Constantine, B., Forget, G., Geib, R., and R. Schrage, "Framework for TCP Throughput Testing", RFC 6349, DOI 10.17487/RFC6349, August 2011.

17.2. Informative References

[RFC7432] Sajassi, A., Ed., et al., "BGP MPLS-Based Ethernet VPN", RFC 7432, DOI 10.17487/RFC7432, February 2015.

[RFC6815] Bradner, S., Dubray, K., McQuaid, J., and A. Morton, "Applicability Statement for RFC 2544: Use on Production Networks Considered Harmful", RFC 6815, DOI 10.17487/RFC6815, November 2012.

[TRAINING-BENCH] "Benchmarking Methodology for AI Training Network Fabrics", draft-bmwg-ai-fabric-training-bench-00, Work in Progress.

[UEC-SPEC] Ultra Ethernet Consortium, "UEC Specification 1.0", 2024.

[VLLM] Kwon, W., et al., "Efficient Memory Management for Large Language Model Serving with PagedAttention", Proceedings of the ACM SIGOPS 29th Symposium on Operating Systems Principles, 2023.

[DISTSERVE] Zhong, Y., et al., "DistServe: Disaggregating Prefill and Decoding for Goodput-optimized Large Language Model Serving", OSDI 2024.

[EP-COMM] "DeepEP: An Efficient Expert-Parallel Communication Library", 2025. (Informative reference to an open-source implementation.)

[KV-DISAGG] "KVCache-centric Disaggregated Architecture for LLM Serving", 2024. (Informative reference.)

[LMCACHE] LMCache Project, "LMCache: Hierarchical KV Cache Management for Inference", 2025.

[DISTRIB-INF] "A Datacenter-Scale Distributed Inference Framework", 2025. (Informative reference.)

[SGLANG] "SGLang: Efficient Execution of Structured Language Model Programs", 2024. (Informative reference.)

[K8S-INF] "llm-d: Kubernetes-Native Distributed LLM Inference", 2025. (Informative reference to an open-source project.)

Appendix A: KPI-to-Test Mapping Summary

The following table provides a cross-reference from each KPI defined in

Section 4 to the test(s) in which it is measured.

+------------------------+------------------------------+------------------+

| KPI                    | Primary Test(s)              | DUT/SUT          |

+------------------------+------------------------------+------------------+

| TTFT                   | 6.1, 6.2, 10.1, 10.3        | SUT-E             |

+------------------------+------------------------------+------------------+

| ITL                    | 10.2, 10.3, 10.4             | SUT-E            |

+------------------------+------------------------------+------------------+

| TPS_output             | 6.2, 11.1, 11.2, 11.3       | SUT-E             |

+------------------------+------------------------------+------------------+

| TPS_input              | 11.1                         | SUT-E            |

+------------------------+------------------------------+------------------+

| Goodput                | 11.1, 11.3                   | SUT-E            |

+------------------------+------------------------------+------------------+

| KV_xfer_latency        | 5.2, 5.3, 6.1, 6.4          | DUT-N, DUT-PD     |

+------------------------+------------------------------+------------------+

| KV_xfer_bandwidth      | 5.1, 5.3, 5.4               | DUT-N, DUT-PD     |

+------------------------+------------------------------+------------------+

| EP_alltoall_latency    | 7.1, 7.2, 7.3, 7.4          | DUT-F             |

+------------------------+------------------------------+------------------+

| EP_alltoall_bandwidth  | 7.1, 7.3                     | DUT-F            |

+------------------------+------------------------------+------------------+

| Fabric_FCT             | 5.2, 5.3                     | DUT-F            |

+------------------------+------------------------------+------------------+

| Buffer_utilization     | 8.1, 8.2                     | DUT-S            |

+------------------------+------------------------------+------------------+

| ECN_marking_rate       | 8.1, 8.3                     | DUT-S            |

+------------------------+------------------------------+------------------+

| PFC_frame_count        | 8.2, 8.4                     | DUT-S            |

+------------------------+------------------------------+------------------+

| Link_utilization       | 5.3, 9.3, 12.1              | DUT-F            |

+------------------------+------------------------------+------------------+

| Packet_drop_rate       | 8.1, 8.2, 12.3              | DUT-F             |

+------------------------+------------------------------+------------------+

| Request_Rate           | 11.1                         | SUT-E            |

+------------------------+------------------------------+------------------+

| Prefix Cache Hit Rate  | 9.2                          | SUT-E            |

+------------------------+------------------------------+------------------+

| JFI (Decode Worker)    | 9.4                          | SUT-E            |

+------------------------+------------------------------+------------------+

Appendix B: Inference Serving Framework Capability Categories (Informational)

This appendix describes the inference serving framework capability categories that are relevant to AI fabric benchmarking. Specific framework names are referenced in informative citations only; this appendix is intended to guide documentation of SUT-E configurations and is NOT normative.

Implementers using a Software Workload Emulator (SUT-E tests) SHOULD document which of the following capabilities their serving framework supports, as these capabilities affect which test categories are applicable and how results should be interpreted.

+------------------------+-----------------------------------+------------------------------+
| Capability             | Description                       | Relevance to                 |
| Category               |                                   | Fabric Benchmarking          |
+------------------------+-----------------------------------+------------------------------+
| Disaggregated          | Physical separation of prefill    | Determines whether           |
| Prefill/Decode (PD)    | and decode execution across       | DUT-PD topology tests        |
|                        | different accelerator pools       | apply (Section 6)            |
+------------------------+-----------------------------------+------------------------------+
| KV Cache Transfer      | Protocol and library used for     | Determines RDMA verb         |
| Protocol               | prefill-to-decode KV state        | types under test and         |
|                        | transfer (one-sided PUT,          | applicable frame formats     |
|                        | two-sided SEND/RECV,              | (Appendix C)                 |
|                        | GPU-initiated)                    |                              |
+------------------------+-----------------------------------+------------------------------+
| MoE Expert             | Distribution of MoE expert        | Determines whether MoE       |
| Parallelism (EP)       | sub-networks across GPUs and      | EP tests apply               |
| Support                | AllToAll dispatch mode support    | (Section 7)                  |
+------------------------+-----------------------------------+------------------------------+
| Continuous             | Dynamic request admission to      | Affects request arrival      |
| Batching               | active inference batches          | rate distributions and       |
|                        |                                   | load balancing tests         |
|                        |                                   | (Section 9)                  |
+------------------------+-----------------------------------+------------------------------+
| Prefix / KV            | Reuse of KV cache segments for    | Determines applicability     |
| Cache Sharing          | requests with common prefixes     | of prefix cache hit rate     |
|                        |                                   | test (Section 9.2)           |
+------------------------+-----------------------------------+------------------------------+
| RDMA Transport         | Underlying transport(s)           | Must be documented;          |
| Support                | supported: RoCEv2, UET,           | affects congestion           |
|                        | or other                          | management test              |
|                        |                                   | interpretation (Section 8)   |
+------------------------+-----------------------------------+------------------------------+
| GPU-Initiated          | Ability for GPU threads to        | Affects RDMA primitive       
| Networking (GIN)       | directly initiate RDMA            | choice in MoE dispatch      
| Support                | operations without CPU            | tests (Section 7)            |
|                        | involvement                       |                              |
+------------------------+-----------------------------------+------------------------------+
| Kubernetes /           | Native support for container-     | Relevant for autoscaling     |
| Orchestration          | based deployment and horizontal   | tests (Section 12.2)         |
| Integration            | scaling                           |                              |
+------------------------+-----------------------------------+------------------------------+
| Maximum Reported       | Maximum cluster scale at which    | Documents applicability      |
| Scale                  | the framework has been validated  | of fabric scale tests        |
+------------------------+-----------------------------------+------------------------------+

Notes: Implementers MUST document the specific framework name, version, and configuration in all test reports. Results obtained with different frameworks are not directly comparable; framework identity is a required reporting parameter per Section 14.

Appendix C: KV Cache Transfer Frame Format

This appendix defines the reference frame formats for KV cache transfer benchmarking over RoCEv2. The frame format follows the standard RoCEv2 encapsulation with one-sided RDMA WRITE (PUT) operations. RoCEv2 KV Cache Transfer Frame (One-Sided RDMA WRITE):

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                Destination MAC Address (bytes 0-3)            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Destination MAC (bytes 4-5)  |   Source MAC Address (0-1)   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                Source MAC Address (bytes 2-5)                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  EtherType = 0x0800 (IPv4) or 0x86DD (IPv6)  | DSCP  |ECN|...|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

|              IPv4/IPv6 Header (20 or 40 bytes)                |

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        Src Port (entropy)     |  Dst Port = 4791 (RoCEv2)    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           UDP Length          |         UDP Checksum          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| OpCode = RDMA_WRITE (0x0A)  |SE|M| Pad |TVer|     PKey       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        Destination QP Number (24 bits)        |A|  Reserved  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           Packet Sequence Number (PSN, 24 bits)               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

|              RETH: Virtual Address (64 bits)                  |

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   Remote Key (R_Key, 32 bits)                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                     DMA Length (32 bits)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

|          KV Cache Payload (variable, up to MTU)               |

|             (key/value attention state data)                  |

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                        ICRC (4 bytes)                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Notes: The UDP Source Port SHOULD use entropy-based values for ECMP load distribution. The RETH (RDMA Extended Transport Header) carries the remote virtual address, remote key, and DMA length for the one-sided

WRITE operation. For KV cache transfers, the DMA Length field indicates the size of the KV cache block being transferred. Typical MTU for RoCEv2 is 4096 bytes; larger KV cache blocks (e.g., 64 KB pages) are segmented

into multiple packets by the NIC. For PUT-with-signal operations, the last packet in the transfer includes an RDMA WRITE with Immediate Data (OpCode 0x0B) to signal completion to the decode worker.

Appendix D: MoE AllToAll Communication Pattern

This appendix describes the AllToAll communication pattern used for MoE expert parallelism dispatch and its fabric-level traffic characteristics. MoE Dispatch AllToAll Traffic Pattern: In a Mixture-of-Experts model with M total experts distributed across N GPUs (each GPU holds M/N experts), a single MoE layer forward pass generates an AllToAll communication pattern where each GPU sends a variable-size payload to every other GPU. The payload to GPU j from GPU i contains the token embeddings routed to experts hosted on GPU j.

Traffic characteristics depend on the dispatch mode:

+------------------------+-------------------------+-------------------------+

| Parameter              | Normal Dispatch         | Low-Latency Dispatch    |

|                        | (Prefill)               | (Decode)                |

+------------------------+-------------------------+-------------------------+

| Batch Size             | 128 - 512 tokens        | 1 - 16 tokens           |

+------------------------+-------------------------+-------------------------+

| Payload per GPU pair   | Variable (depends on    | Fixed (padded to max)   |

|                        | routing)                |                         |

+------------------------+-------------------------+-------------------------+

| Shape Compatibility    | Dynamic (symbolic)      | Static (CUDA Graph)     |

+------------------------+-------------------------+-------------------------+

| QP Parallelism         | 24 QPs per connection   | 8 - 16 QPs per          |

|                        |                         | connection              |

+------------------------+-------------------------+-------------------------+

| RDMA Primitive         | Two-sided SEND/RECV or  | One-sided PUT           |

|                        | one-sided PUT           | (GPU-direct RDMA, GIN)  |

+------------------------+-------------------------+-------------------------+

| GPU Initiation         | CPU-initiated or GIN    | GIN (device-initiated,  |

|                        |                         | GPU-to-NIC direct)      |

+------------------------+-------------------------+-------------------------+

| Typical per-dispatch   | 1 - 10 MB aggregate     | 10 KB - 1 MB aggregate  |

| size                   |                         |                         |

+------------------------+-------------------------+-------------------------+

| Dispatch Frequency     | Once per MoE layer      | Once per MoE layer per  |

|                        | (prefill)               | token (decode)          |

+------------------------+-------------------------+-------------------------+

| Latency Target         | < 1 ms per dispatch     | < 200 us per dispatch   |

+------------------------+-------------------------+-------------------------+

For a representative large-scale MoE model (example: 256 experts, top-2 routing, hidden_dim=7168, EP=96 across 12 nodes), the inter-node traffic per MoE layer dispatch is approximately: Normal Dispatch (prefill, batch=256): 256 * 2 * 7168 * 2 bytes / 96 GPUs = \~76 KB per GPU pair, \~870 MB aggregate across all pairs. Low-Latency Dispatch (decode, batch=8): 8 * 2 * 7168 * 2 bytes / 96 GPUs = \~2.4 KB per GPU pair, \~27 MB aggregate.

With 61 MoE layers and a decode iteration time target of \~30 ms, the\ decode phase requires 61 AllToAll dispatches within 30 ms, yielding \~2,000 dispatches per second per decode step, consuming approximately 54 GB/s aggregate inter-node bandwidth for the Low-Latency Dispatch path.

Authors' Addresses

Fernando Calabria
Cisco
Email: fcalabri@cisco.com

Carlos Pignataro
Blue Fern Consulting
Email: carlos@bluefern.consulting

Contributions and review are solicited from the BMWG mailing list (bmwg@ietf.org) and the broader AI networking community. The BMWG chairs and Area Director are identified at https://datatracker.ietf.org/group/bmwg/about/.